# Lab 2.2 Report

- Problem analysis

Lab 2.2 is the next episode of lab 2.1. In lab 2.1, we have simulated the assembly process that turns the assembly code into binary machine code. And in lab 2.2, we are going to keep making use of these machine codes and perform real instructions, like changing the registers and data memory, simulating the real instruction execution process.

- My understanding of the instruction execution process

By examining the sim.c and sim.h files, I have a basic understanding about the whole instruction execution process.

- Terminologies and data structures

- System latches

In my understanding, system latches is the system of the combination of the 32 registers and the PC counter. It is just the place to store information in the processor. System latches is simulated by the structure struct_system_latches, where it has 2 members: integer PC that stores the value of the PC and integer array REGS that stores all the register value of the 32 registers.

- Special base addresses

CODE_BASE_ADDR: the base address to locate source codes.

TRAPVEC_BASE_ADDR: exceptions & interruptions' base address.

This is one of the important things I learned from this lab: in practice, actually exceptions and interruptions are stored in their unified position in memory, and if the pc just to that address, the system will know there is an error.

- Memory

The simulator uses C arrays to simulate the memory. It is an array named MEMORY that has limited total amount BYTES_IN_MEM and is of type unsigned char, which is of I byte that is the same with the real situation.

- Important functions and MACROS

- The MASK macros. In this simulating code, we need to frequently retrieve some specific bits of the instructions. In order to avoid tedious and unclear bit wise and operation everywhere, we can use a macro to clear the things. A typical MASK is like this:

#define MASK6_0(x) ((x) & 0x7F)

Where the return value is the input bitwise and a masking value, so all the bits in the masking area remain and all other bits are set to 0 due to the property of and logic.

- Simulation process

The simulator is actually a user-interact system. After running this file, we are actually in a user interface where we can let the simulator execute the binary machine code and examine the registers and memories.

Here are the possible instructions:

- Program logic:

The program starts with the main function. In the main function, the program first get the binary file from the console and open it, then it starts to initialize like allocating space for memory, etc. Then it goes into a while loop in which the get_command function is executed and will get command from the user interface. It can show the values in specific memories or registers, and it can also execute instructions.

The execution process is handled by the run functions. It runs a specific numbers of instructions by calling the cycle() function a specific amount of times, where each time the cycle() function executes exactly one instruction.

In the cycle instruction, the status of instruction count and latches is updated, and each instruction is handled by the handle_instruction() function, which is what we should implement.

In handle_instruction() function, it first does some previous preparations like updating pc, then it uses a switch logic to identify which instruction the current cycle is by examining the opcode, func3, func7, etc. In lab 2.2 we should implement the details of handle_instruction() function, including the switch logic and the function for handling specific instructions.

- Switch logic implementation

The instruction identifying logic (decoding) in handle_instruction() function is done by a nested branching. On the outmost side is the switch function to decide on the general types of instruction (I, halt, R, U, jal, halr, B, load and store) by its opcode. On the second stage of the decoding, a nested switch is implemented to identify the specific instructions by func3. And if we still need to identify specific instructions by the 25..31 bits, a if_else logic could also be implemented. In the given code, the I and halt type logic is already implemented, so we need to finish the rest.

- R type instructions

The opcode of R type instructions is 0110011, which can be done by shifting 0x0C left 2 bits and add 0x03. And this number is put into the outmost switch logic to identify this type of instruction.

After knowing the type of instructions, we still have to examine func3 using another nested switch to know which instructions it is. Possible cases are 0, 1, 4, 5, 6, 7, corresponding to add/sub, sll, xor, srl/sra, or and respectively.

For add/sub and srl/sra, we also have to check the 25..31 bits to get the specific instructions. This can be done by using MASK31_25 to fetch the 25..31 bits and use an if_else to check which value it is. If it is 0, then the instruction should be add or srl according to func3,  and otherwise it should be sub or sra according to func3.

Inside each instruction cases, handle_instruction() call the specific handle_xxx() function to execute each specific functions, which will be discussed in the next section.

If none of the switch cases matches, then there should be some mistakes in the input file, this error is handled by the error function.

- U type

The U type instruction here is only lui, so we don’t need so many branching logics.

We just test if the opcode is 0110111, which can be done by shifting 0x0D left 2 bits and add 0x03. If it is, handle_instruction() call the specific handle_lui() function to execute specific function, which will be discussed in the next section.

- jalr

The opcode is 1100111, which can be done by shifting 0x19 left 2 bits and add 0x03. It is unique so we don’t need so many branching logics. We just test if the opcode is 1100111, If it is, handle_instruction() call the specific handle_jalr() function to execute specific function, which will be discussed in the next section.

- jal

The opcode is 1101111, which can be done by shifting 0x1b left 2 bits and add 0x03. It is unique so we don’t need so many branching logics. We just test if the opcode is 1101111, If it is, handle_instruction() call the specific handle_jal() function to execute specific function, which will be discussed in the next section.

- B type

The opcode of B type instructions is 1100011, which can be done by shifting 0x18 left 2 bits and add 0x03. And this number is put into the outmost switch logic to identify this type of instruction.

After knowing the type of instructions, we still have to examine func3 using another nested switch to know which instructions it is. Possible cases are 0, 1, 4, 5 corresponding to beq, bne, blt, bge respectively.

Inside each instruction cases, handle_instruction() call the specific handle_xxx() function to execute each specific functions, which will be discussed in the next section.

If none of the switch cases matches, then there should be some mistakes in the input file, this error is handled by the error function.

- Load instructions

The opcode of store type instructions is 0000011, which is 0x03. And this number is put into the outmost switch logic to identify this type of instruction.

After knowing the type of instructions, we still have to examine func3 using another nested switch to know which instructions it is. Possible cases are 0, 1, 2 corresponding to lb, lh, lw respectively.

Inside each instruction cases, handle_instruction() call the specific handle_xxx() function to execute each specific functions, which will be discussed in the next section.

If none of the switch cases matches, then there should be some mistakes in the input file, this error is handled by the error function.

- store instructions

The opcode of store type instructions is 0100011, which can be done by shifting 0x08 left 2 bits and add 0x03. And this number is put into the outmost switch logic to identify this type of instruction.

After knowing the type of instructions, we still have to examine func3 using another nested switch to know which instructions it is. Possible cases are 0, 1, 2 corresponding to sb, sh, sw respectively.

Inside each instruction cases, handle_instruction() call the specific handle_xxx() function to execute each specific functions, which will be discussed in the next section.

If none of the switch cases matches, then there should be some mistakes in the input file, this error is handled by the error function.

The code is shown in the main code section.

- Handle specific instructions

In handle_xxx() functions we perform the decode, exe, wb of each instructions.

- Handle shift immediates

This includes handle_slli, handle_srli, handle_srai. In these functions we have to decode rd, rs1, calculate immediate value, and perform exe and wb by calculating the result and update the system latches.

The rd can be decoded by first retrieving the 7..11 bits using MASK11_7, rs1 can be decoded by first retrieving the 15..19 bits using MASK19_15.

The immediate value is 5 bits so can be retrieve by masking the 20..24 bits with the MASK24_20. Since it is not signed value, so we DO NOT need to use the sext() function to perform sign extend, to extend the 5 bits into 32 bits.

The result is calculated by shifting CURRENT_LATCHES.REGS[rs1] by the immediate value, and assign it to NEXT_LATCHES.REGS[rd], so this register will be updated after the this cycle is finished.

- Handle immediate logic

This includes handle_xori, handle_ori, handle_andi. In these functions we have to decode rd, rs1, calculate immediate value, and perform exe and wb by calculating the result and update the system latches.

The immediate value is 12 bits so can be retrieve by masking the 20..31 bits with the MASK31_20. Since it is signed value and we need to extend the 12 bits into 32 bits, we have to use the sext() function to perform sign extend.

The rd can be decoded by first retrieving the 7..11 bits using MASK11_7, rs1 can be decoded by first retrieving the 15..19 bits using MASK19_15.

The result is calculated by performing logic operation on the  CURRENT_LATCHES.REGS[rs1] with the immediate value, and assign it to NEXT_LATCHES.REGS[rd], so this register will be updated after the this cycle is finished.

- Handle lui

In handle_lui we have to decode rd, calculate immediate value, and load the immediate value to upper 20 bits of rd.

The rd can be decoded by first retrieving the 7..11 bits using MASK11_7.

The immediate value is 20 bits so can be retrieve by masking the 12..31 bits with the MASK31_20. Since it is signed value and we need to extend the 20 bits into 32 bits, we have to use the sext() function to perform sign extend.

To load the value into upper 20 bits of rd, we need to update NEXT_LATCHES.REGS[rd] to the immediate value left shifted 12 bits.

- Handle register-register arithmetic

This includes handle_add, handle_sub. In these functions we have to decode rd, rs1, rs2, and perform exe and wb by calculating the result and update the system latches.

The rd can be decoded by first retrieving the 7..11 bits using MASK11_7, rs1 can be decoded by first retrieving the 15..19 bits using MASK19_15, rs2 can be decoded by first retrieving the 20..24 bits using MASK24_20.

The result is calculated by performing arithmetic operation on the  CURRENT_LATCHES.REGS[rs1] with CURRENT_LATCHES.REGS[rs2], and assign it to NEXT_LATCHES.REGS[rd], so this register will be updated after the this cycle is finished.

- Handle register-register shift

This includes handle_sll, handle_srl, handle_sra. In these functions we have to decode rd, rs1, rs2, and perform exe and wb by calculating the result and update the system latches.

The rd can be decoded by first retrieving the 7..11 bits using MASK11_7, rs1 can be decoded by first retrieving the 15..19 bits using MASK19_15, rs2 can be decoded by first retrieving the 20..24 bits using MASK24_20.

The result is calculated by performing shifting the  CURRENT_LATCHES.REGS[rs1] with the amount CURRENT_LATCHES.REGS[rs2], and assign it to NEXT_LATCHES.REGS[rd], so this register will be updated after the this cycle is finished.

- Handle register-register logic

This includes handle_xor, handle_or, handle_and.  In these functions we have to decode rd, rs1, rs2, and perform exe and wb by calculating the result and update the system latches.

The rd can be decoded by first retrieving the 7..11 bits using MASK11_7, rs1 can be decoded by first retrieving the 15..19 bits using MASK19_15, rs2 can be decoded by first retrieving the 20..24 bits using MASK24_20.

The result is calculated by performing logic operation on the  CURRENT_LATCHES.REGS[rs1] with CURRENT_LATCHES.REGS[rs2], and assign it to NEXT_LATCHES.REGS[rd], so this register will be updated after the this cycle is finished.

- jalr

In handle_jalr we have to decode rd, rs1, calculate immediate value, and update pc and rd.

The rd can be decoded by first retrieving the 7..11 bits using MASK11_7. rs1 can be decoded by first retrieving the 15..19 bits using MASK19_15.

The immediate value is 12 bits so can be retrieve by masking the 20..31 bits with the MASK31_20. Since it is signed value and we need to extend the 12 bits into 32 bits, we have to use the sext() function to perform sign extend.

To make sure that we can jump back, we should update NEXT_LATCHES.REGS[rd] to CURRENT_LATCHES.PC +4, which is the address of the next instruction.

To jump, we should update NEXT_LATCHES.PC to the sum of CURRENT_LATCHES.REGS[rs1] and the immediate value. So that the next instruction will be the place to branch to.

- jal

In handle_jal we have to decode rd, calculate immediate value, and update pc and rd.

The rd can be decoded by first retrieving the 7..11 bits using MASK11_7.

The immediate value is 21 bits and the order is mixed, so we have to retrieve it part by part and perform adding and shifting. The bit 0 is always 0 so we can ignore it. The bit 1..10 is at bit 21..30 of the instruction, so can be retrieved by masking the 21..30  bits with the MASK30_21 and shift it 1 bit and add it to the binary value. The bit 11 is at bit 20 of the instruction, so can be retrieved by masking the 20th bit with the MASK20 and shift it 11 bits and add it to the binary value. The bit 12..19 is at bit 12..19 of the instruction, so can be retrieved by masking the 12..19 bits with the MASK19_12 and shift it 12 bits and add it to the binary value. The bit 20 is at bit 31 of the instruction, so can be retrieved by masking the 31 bit with the MASK31 and shift it 31 bits and add it to the binary value.

To make sure that we can jump back, we should update NEXT_LATCHES.REGS[rd] to CURRENT_LATCHES.PC +4, which is the address of the next instruction.

To jump, we should update NEXT_LATCHES.PC to the sum of CURRENT_LATCHES.REGS[rs1] and the immediate value sign extended to 32 bits (similar to I-type instructions, the immediate could also be negative). So that the next instruction will be the place to branch to.

- Handle branches

This includes handle_beq, handle_bne, handle_blt, handle_bge.  In these functions we have to decode rs1, rs2, calculate immediate value, test the condition and update pc.

The rs1 can be decoded by first retrieving the 15..19 bits using MASK19_15, rs2 can be decoded by first retrieving the 20..24 bits using MASK24_20.

The immediate value is 13 bits, and the order is mixed, so we have to retrieve it part by part and perform adding and shifting. The bit 0 is always 0 so we can ignore it. The bit 1..4 is at bit 8..11 of the instruction, so can be retrieved by masking the 8..11  bits with the MASK11_8 and shift it 1 bit and add it to the binary value. The bit 5..10 is at bit 25..30 of the instruction, so can be retrieved by masking the 20..30 bit with the MASK30_25 and shift it 5 bits and add it to the binary value. The bit 11 is at bit 7 of the instruction, so can be retrieved by masking the 17th bits with the MASK7 and shift it 11 bits and add it to the binary value. The bit 12 is at bit 31 of the instruction, so can be retrieved by masking the 31st bits with the MASK31 and shift it 12 bits and add it to the binary value.

Then we need to test the conditions based on CURRENT_LATCHES.REGS[rs1] and CURRENT_LATCHES.REGS[rs2]. If the condition is met, we need to jump, so we should update NEXT_LATCHES.PC to the sum of CURRENT_LATCHES.PC and the immediate value sign extended to 32 bits (similar to I-type instructions, the immediate could also be negative). So that the next instruction will be the place to branch to.

- Load

This includes handle_lb, handle_lh, handle_lw.  In these functions we have to decode rd, rs1, calculate immediate value, and update rd by the value retrieved from memory.

The rd can be decoded by first retrieving the 7..11 bits using MASK11_7. rs1 can be decoded by first retrieving the 15..19 bits using MASK19_15.

The immediate value is 12 bits so can be retrieve by masking the 20..31 bits with the MASK31_20. Since it is signed value and we need to extend the 12 bits into 32 bits, we have to use the sext() function to perform sign extend.

To retrieve the value from memory, we need to access the MEMORY array. For different instructions, we might need to access MEMORY with different offsets.

Here I take lw as an example since it needs to access biggest amount of memory and all other instructions memory access is included in lw. To access the nth (n is 0, 1, 2, 3) byte starting from the given address calculated by sext(imm12, 12) + CURRENT_LATCHES.REGS[rs1], we can use MEMORY[sext(imm12, 12) + CURRENT_LATCHES.REGS[rs1] + n] to get the value. Then we need to left shift it n*8 bits to its corresponding position and add it to the NEXT_LATCHES.REGS[rd].

- Store

This include handle_sb, handle_sh, handle_sw.  In these functions we have to decode rs1, rs2, calculate immediate value, and store the value to memory.

The rs1 can be decoded by first retrieving the 15..19 bits using MASK19_15, rs2 can be decoded by first retrieving the 20..24 bits using MASK24_20.

The immediate value is 12 bits but it is separated to 2 parts, so we have to retrieve it part by part and perform adding and shifting. The bit 0..4 is at bit 7..11 of the instruction, so can be retrieved by masking the 7..11  bits with the MASK11_7. The bit 5..11 is at bit 25..31 of the instruction, so can be retrieved by masking the 25..31  bits with the MASK31_25 and shift it 5 bit and add it to the binary value. Finally a sign extension is applied.

To store the value to memory, we need to access the MEMORY array. For different instructions, we might need to access MEMORY with different offsets.

Here I take sw as an example since it needs to access the biggest amount of memory and all other instructions memory access is included in sw. To access the nth (n is 0, 1, 2, 3) byte starting from the given address calculated by sext(imm12, 12) + CURRENT_LATCHES.REGS[rs1], we can write each bytes of the value to MEMORY[sext(imm12, 12) + CURRENT_LATCHES.REGS[rs1] + n]. The byte 1, 2, 3, 4 of the immediate is gotten by MASK7_0, MASK15_8, MASK23_16 and MASK31_24 of the value CURRENT_LATCHES.REGS[rs2].

The code is shown in the main code section.

- My mistakes

During coding, I made some mistakes and I think it is worthwhile to be recorded in the lab report.

- The memory access is by bytes.

At first when I was performing at first wasn’t by bytes, but is just assigning or  retrieving the whole 32 bits of the memory altogether using a MEMORY[sext(imm12, 12) + CURRENT_LATCHES.REGS[rs1]]. I have bebugged for a long time. I think the reason to make this mistake is that I haven’t read the code entirely and didn’t notice that memory array is of type unsigned char which is 8 bits.

- PC update

To update the next PC, for example in jal, I initially just updated it on CURRENT_LATCHES.PC, ie. CURRENT_LATCHES.PC += sext(imm21, 21). Then executing the isa.bin, the program goes to a infinite loop. Then I found that this is because we should update it on the next PC, ie. NEXT_LATCHES.PC = CURRENT_LATCHES.PC + sext(imm21, 21). This is because in the implementation, before the instruction is handled, the NEXT_LATCHES.PC is already updated to CURRENT_LATCHES.PC+4 and will no longer update if no special branch. So updating CURRENT_LATCHES.PC is of no use. This can be understood with the 5 stages of instructions: pc+4 is done in the instruction fetch stage, which is before, EXE and WB.

- Main code:

- Handle_instruction

- Hande_xxx

- Console results

- Make:

There are some warnings but it turns out to be some version issues and is about the unsafe use of some string manipulation functions, so we can ignore it in this assignment.

- Isa.bin

- Count10

- Swap

- Add4

# Reference:

TextBook -Computer Organization and Design_ The Hardware Software Interface [RISC-V Edition]

opcodes-rv32i reference document

risc-v-asm-manual.pdf

riscv-spec-20191213.pdf